# PicoRV32A Synthesis using OpenLANE (Sky130)

## Overview

This project documents the synthesis phase of the picorv32a RISC-V core using the OpenLANE ASIC flow with the Sky130 PDK.

The goal was not just to run commands blindly, but to understand:

- how OpenLANE organizes a design run

- how synthesis transforms RTL into gate-level hardware

- how reports are generated

- how timing and area are analyzed after synthesis

This repository captures screenshots, reports, and observations collected during the synthesis stage.

# Environment Setup

The OpenLANE container was launched and the design was prepared interactively.

## Design Preparation

### Observation

During preparation:

- OpenLANE loaded the picorv32a configuration

- Sky130A PDK was initialized

- Standard cell library sky130_fd_sc_hd was selected

- Run directory was automatically created

One interesting thing noticed here was that OpenLANE internally creates a timestamped run folder for every execution.
This makes experiments reproducible and prevents overwriting previous runs.

Screenshot:

1_design_preparation.png

# Run Directory Structure

## Generated Run Directory

### Observation

The run directory contained:

- logs/

- reports/

- results/

- tmp/

This separation makes debugging easier because:

- logs show execution flow

- reports contain analysis

- results contain generated design files

- tmp stores intermediate files

It felt surprisingly organized for such a complex ASIC flow

Screenshot:

2_run_directory_structure.png

# Running Synthesis

## Synthesis Execution

### Observation

The synthesis stage converts RTL Verilog into:

- logic gates

- flip-flops

- buffers

- optimized netlists

OpenLANE also automatically performed:

- single-corner STA

- timing checks

- optimization

At this point, the design stops looking like “code” and starts looking like actual hardware.

Screenshot:

3_run_synthesis.png

# Synthesis Reports

## Generated Reports

### Observation

The synthesis reports included:

- area reports

- DFF statistics

- timing reports

- synthesis statistics

This showed how much information ASIC tools generate automatically after compilation.

Screenshot:

4_synthesis_reports.png

# Cell Count Analysis

## Synthesized Cell Statistics

### Observation

The synthesized design contained:

- Total Cells: 15762

A large number of logic cells were inserted automatically during synthesis.

Interesting observation:
many simple RTL operations expand into hundreds or thousands of actual gates after optimization.

The design heavily used:

- XOR gates

- OR gates

- combinational logic

- buffers

This gave a better appreciation for how large even a “small” processor really becomes at gate level.

Screenshot:

5_synthesis_cellcount.png

# Chip Area Report

## Area Estimation

### Observation

Reported chip area:

160816.736000

This value represents the estimated silicon area occupied by the synthesized design.

One important realization here:
optimization is not only about functionality, but also:

- area

- timing

- power

- routability

ASIC design is basically controlled engineering chaos wrapped in reports.

Screenshot:

6_synthesis_area.png

# Flip-Flop Analysis

## DFF Count Report

### Observation

The design contained:

- DFF Count: 1613

Flop ratio calculation:

1613/18508 = 0.0871

Approximately 8.7% of the total cells are sequential elements.

This indicates the processor contains a significant amount of:

- state storage

- pipelined/control logic

- sequential circuitry

The remaining cells are mostly combinational logic

Screenshot:

7_dff_count_report.png

# Pre-Synthesis Statistics

## Initial Synthesis Statistics

### Observation

Before technology mapping:

- the design was represented using generic logic cells

- synthesis optimization reduced and transformed the logic

Comparing pre-synthesis and mapped reports helped visualize how synthesis reshapes the hardware internally.

Screenshot:

8_pre_synthesis_stats.png

# Timing Analysis

## Timing Summary Report

### Key Results

TNS = 0.00
WNS = 0.00
Worst Setup Slack = 0.52
Worst Hold Slack = 0.16

### Observation

This was probably the most satisfying part of the run.

The synthesized design passed timing successfully:

- no setup violations

- no hold violations

The positive slack values indicate that the design meets timing requirements under the analyzed corner.

This is where ASIC design suddenly feels very real.

Screenshot:

9_timing_summary_report.png

# Post-Synthesis Netlist

## Gate-Level Netlist

### Observation

The RTL was transformed into real standard cells such as:

sky130_fd_sc_hd__buf_2

This gate-level netlist represents actual hardware components that can eventually be placed and routed physically.

At this stage:

- the design is no longer behavioral RTL

- it becomes manufacturable logic representation

Seeing the processor reduced into thousands of standard cells was honestly one of the coolest moments in the flow.

Screenshot:

10_post_synthesis_netlist.png

# Final Thoughts

This synthesis run helped in understanding:

- ASIC flow structure

- OpenLANE workflow

- synthesis reports

- timing analysis

- gate-level hardware representation

Biggest takeaway:

Writing Verilog is only the beginning.
The real story starts when the tools begin transforming it into physical hardware.

# Tools Used

- OpenLANE

- OpenROAD

- Sky130A PDK

- PicoRV32A RISC-V Core

- VS Code Codespaces

- Docker